MODULE 4

HAZARDOUS SUBSTANCES

- INDIVIDUALS

- ENVIRONMENT

Project: 2013-1-BG1-LEO05-088722

Author:	Springboard Opportunities, Northern Ireland, UK

October, 2014

This project has been funded with support from the European Commission. This publication reflects the views only of the author, and the Commission cannot be held responsible for any use which may be made of the information contained therein.

TABLE OF CONTENTS

OVERVIEW	3

Introduction – What are hazardous substances?	3

Chemical substances	4

Basic Safety Measures	5

Highly flammable chemicals	6

HAZARDOUS SUBSTANCES	7

Physical hazards pictograms	9

Health hazards pictograms	12

Environmental hazards pictograms	15

Safety Labels	15

Protecting others and the environment	17

What you need to do	17

Materials storage	17

Effects on the environment	18

Measures to Prevent Pollution	19

# OVERVIEW

## Introduction – What are hazardous substances?

Any substance that can cause harm or illness to a worker can be deemed as hazardous.  Thousands of workers each year are made ill by these substances, costing millions to the industry and the individuals. The best way to prevent this is to follow employer health and safety procedures, carry out risk assessments and wear appropriate personal protective equipment.

This module will help you to learn about and identify some of the more common hazardous substances found on construction sites and also look at the EU chemical labelling standards that will be found on these substances containers.

We will also explore the problems hazardous substances and the construction site in general can have an impact on the environment.

Once you complete your study of this module there will be a short multiple choice quiz at the end.

# Chemical substances

A great many chemical substances are used in construction – there is hardly a site without them. They are found in adhesives, cleaning agents, fungicides, grouts, cements, insulation materials, sealants, paints, solvents and many more.

#### Chemical risks

Many chemicals are hazardous and have the potential to cause fires and explosions, or they are toxic, with the potential to cause poisoning leading to effects, such as dizziness, vomiting and headaches. Other chronic effects resulting from high levels of exposure or long term exposure can lead to lung and respiratory diseases. When not using the proper safety equipment or through bad handling of chemicals workers can also be in danger of dermatological (hair, nails and skin) issues resulting from the contact with some chemicals. Acids and alkalis are corrosive and can damage the skin and eyes, some even with the use of protective equipment and tools.

#### Entry into the body

A chemical can cause injury whether it is solid, liquid, dust, vapour, fumes or gas. The routes into your body are by:

Inhalation or breathing in. This is the most common route of entry. Toxic gases or vapours can cause irritation in the nose and throat, this is the first sign that something is wrong; other gases or vapours do not cause this irritation but can be just as toxic, they can penetrate to the lungs or blood stream before you even know there is a problem. Inhaled dust, as seen in our picture, gathers in the lungs and can lead to shortness of breath, coughing and possible lung diseases. Some dusts such as asbestos destroy the lung tissue and may lead to the development of tuberculosis or cancer. It is also worth noting that dust does not have to be seem to be harmful, it is often the small dust particles we cannot see that do the most serious damage.

Absorption through the skin. Some solvents can be absorbed through the skin into the blood stream and travel to internal organs such as the brain and liver. Contact dermatitis or eczema results from skin contact with some chemicals, like our picture here with an unprotected worker using cement. Acids and alkalis are corrosive and can damage the skin and the eyes on contact. Unless large amounts of water are used at once to rinse the substance off, serious burns will be caused.

Ingestion or swallowing. This is possible when you handle chemicals or chemical containers and then eat or smoke without washing your hands or removing gloves. This can also be an issue if you are handling materials such as insulation foam, paints, solvents and cement dust to name but a few. As demonstrated in our picture it may not only be you who is at risk, here a worker is feeding some birds while still wearing PPE clothing.

#### Prevention

Accidents and ill health from the use of chemicals can be prevented or at least the potential for danger can be minimised through basic precautions such as:

- Knowing what chemicals you are using and the risks they pose,

- Follow the established safe practice in handling them,

- Substitute the chemical with a harmless or less hazardous one,

- Enclose the process using the chemical, or provide other controls such as exhaust ventilation,

- Use personal protective equipment (PPE).

# Basic Safety Measures

If the use of hazardous chemicals cannot be avoided, here are some basic safety measures you can adopt to protect yourself and others from danger:

- Keep containers of chemicals in a separate and secure store.

- Make sure there is a label on the container – if there is no label, then do not use the contents and report this to a supervisor or store person responsible.

- Always read the label and make sure you understand what it says, then follow the instructions.

- If the information is not sufficient to tell you how to handle the chemical safely, ask for the chemical safety data sheet do not use the chemical until you have seen it; if you do not understand it, ask questions until you do.

- Check that you are wearing the correct PPE before you handle chemicals and that the equipment is not damaged.

- Avoid breathing in any fumes from chemicals. Provide good ventilation, or work in the open air. Leave the work area immediately if you feel dizzy or unwell.

- If you are using large quantities of solvents, wear PPE. Remove any clothing that comes into contact with solvents and leave it to dry in a well-ventilated place.

- Use the smallest quantity of chemicals that is necessary for the particular job.

- Always wear eye protection when chemicals are being moved or transferred on site.

- When mixing or pouring chemicals using temporary containers, make sure they are suitable and correctly labelled. Never use food or drink containers as this increases the risk of an individual ingesting the chemical.

- Wash thoroughly before you eat and do not eat or smoke at your workstation.

- If skin is splashed with a chemical, it should be rinsed immediately with plenty of clean running water. Eyes should be flushed out thoroughly with water and should receive immediate medical attention.

- If you are burned by a chemical, or feel unwell after using a chemical, seek medical attention immediately.

- If there is a spillage of chemicals on the ground or floor, report the matter at once so that the right action can be taken, such as soaking it up with dry sand or absorbent pads.

# Highly flammable chemicals

Many chemical substances used in construction are highly flammable as well as toxic. The following precautions should be followed when handling or using them:

- Keep containers in the store until they are required for use, and return them there when you have finished with them. Store drums upright.

- Study the label and the instructions on the chemical safety data sheet about safe handling and first-aid measures.

- Use funnels and spouts to prevent spillage. Soak up any spillage with dry sand and remove the contaminated sand to a safe disposal site.

- Always transfer the contents of large containers to small containers in the open air and into an appropriate container for the task, never use food or drink containers.

- Remember that all flammable liquids give off vapours which travel unseen into the air and are easily ignited. Never smoke if there are flammable chemicals in the area.

- If you cannot avoid using highly flammable liquids in an enclosed area, make sure there is an adequate supply of fresh air. This can usually be achieved by opening windows and doors to the full. If it is necessary to use a fan, check that it is electrically safe to use in a flammable atmosphere and may not cause a higher risk.

- Treat empty drums with as much care as unused ones – they will still contain flammable vapour and may still have some liquid.

# HAZARDOUS SUBSTANCES

#### Asbestos

Although no longer used in construction within the EU asbestos can still be found in buildings from pre 2000. Breathing in asbestos dust can kill by causing irreversible lung damage and cancer. There is no known cure for most asbestos related diseases. The more asbestos dust breathed in, the greater the risk to health. There are control limits for the various types of asbestos and specialist training may be available through your employer.

If you suspect asbestos in your work area ensure you have notified your supervisor or the relevant authorities. Unless you are specifically trained in asbestos removal and have the necessary storage, do not attempt to remove asbestos yourself.

You are likely to find asbestos in the following situations:

(1) as insulation or coating asbestos was used for:

- thermal insulation of boilers;

- fire protection of structural steelwork;

- thermal and acoustic insulation of buildings;

(2) as an insulating board asbestos was used in a wide variety of places such as:

- fire protection on doors, protected exits, structural steelwork, etc.;

- cladding on walls, ceilings, etc.;

- internal walls and partitions;

- suspended ceiling tiles;

(3) as asbestos cement, which can be found in:

- corrugated sheets (roofing and cladding of buildings);

- flat sheeting for partitioning, cladding and door facings;

- gutters and downpipes.

Please click here to see a gallery of asbestos related images from the UK http://www.hse.gov.uk/asbestos/gallery.htm

#### Cement

Cement mixes are a well-known cause of skin disease. Both irritant and allergic contact dermatitis can result from proximity to wet cement. Prolonged exposure to wet cement (for example, if you kneel or stand in it) may cause cement burns or ulceration of the skin. The following precautions should be taken:

- Avoid breathing in cement dust, as well dust created by the surface treatment of hardened concrete which may contain a high silica content, by wearing suitable respiratory protective equipment.

- Protect the skin from contact by wearing long-sleeved clothing and full-length trousers, with rubber boots and gloves when required.

- Protect the eyes; if any cement gets into the eyes, rinse them immediately with plenty of warm water.

- Immediately wash off any dust or freshly mixed cement that gets on to the skin.

- Clean off your clothing and boots after work.

#### Lead

Inorganic lead is found in many construction products, for example electricity cables, pipes, gutters and old lead sheet roofs. Organic lead is added to motor fuels, and storage tanks will be heavily contaminated.

There is a risk to health from inhaling dust or fumes created by burning or cutting materials containing lead, including painted surfaces, by welding, by grinding or cutting, and by spray painting of leaded paints. Lead can be absorbed when swallowed, usually when food is contaminated, and adequate washing facilities should be provided. Organic lead compounds are readily absorbed through the skin.

Excessive lead absorption causes constipation, abdominal pain, anaemia, weak muscles and kidney damage. It can also affect the brain, causing impaired intellect, strange behaviour, fits and coma. If you work with lead in any form, you should take the following precautions:

- Wash your hands regularly, and always before eating; you are at higher risk if you smoke with lead on your hands.

- Use the protective clothing and respiratory protective equipment which should be provided whenever lead levels exceed national control limits.

- Wear work clothing on the job and store your “street” clothing where it cannot be contaminated by your work clothing.

Help in managing these risks

To help manage and better identify the risks mentioned above, the EU has worked with and joined the Globally Harmonized System of Classification and Labeling of Chemicals (GHS), an internationally agreed-upon system, created by the United Nations. The European Union CLP Regulation (Classification, Labelling and Packaging) was introduced through the European directives (EC) N°1272/2008 and is based on the GHS system, to secure for physical, health and environment hazards. European Regulation (EC) No 1272/2008 on classification, labelling and packaging of substances and mixtures came into force on 20 January 2009 in all EU Member States. It is known by its abbreviated form, ‘the CLP Regulation’ or just plain ‘CLP’.

Two sets of pictograms are included within the GHS: one for the labelling of containers and for workplace hazard warnings, and a second for use during the transport of dangerous goods. Either one or the other is chosen, depending on the target audience but the two are not to be used together.

The pictograms use the same symbols for the same hazards, thereby ensuring the warnings are recognisable in multiple countries and by multiple nationalities.

Hazard pictograms are one of the key elements for the labelling of containers under the GHS, along with:

- an identification of the product; a signal word – either DANGER (for more severe hazard categories) or WARNING (for less severe hazard categories)– where necessary

- hazard statements, indicating the nature and degree of the risks posed by the product

- precautionary statements, indicating how the product should be handled to minimize risks to the user (as well as to other people and the general environment)

- the identity of the supplier (who might be a manufacturer or importer)

We will now look at the main 9 pictograms from the CLP regulations

## Physical hazards pictograms

| EXPLOSIVE | What does it mean? Unstable explosive Explosive; mass explosion hazard Explosive; severe projection hazard Explosive; fire, blast or projection hazard May mass explode in fire Warning label examples Obtain special instructions before use Do not handle until all safety precautions have been read and understood Keep away from heat/sparks/open flames/hot surfaces. – No smoking Wear protective gloves/protective clothing/eye protection/face protection Use personal protective equipment as required Explosion risk in case of fire Examples of where you would find it Fireworks, ammunition |
| --- | --- |
| Old Symbol |  |

| FLAMMABLE | What does it mean? Extremely flammable gas Flammable gas Extremely flammable aerosol Flammable aerosol Highly flammable liquid and vapour Flammable liquid and vapour Flammable solid Warning label examples Do not spray on an open flame or other ignition source. Keep away from heat/sparks/open flames/hot surfaces – No smoking Keep container tightly closed Keep cool Protect from sunlight Examples of where we can find it Lamp oil, petrol, nail polish remover |
| --- | --- |
| Old Symbol |  |

| OXIDIZING | What does it mean? May cause or intensify fire; oxidiser. May cause fire or explosion; strong oxidiser. Warning label examples Keep away from heat/sparks/open flames/hot surfaces. – No smoking Wear protective gloves/protective clothing/eye protection/face protection. Rinse immediately contaminated clothing and skin with plenty of water before removing clothes. Examples of where we can find it Bleach, oxygen for medical purposes |
| --- | --- |
| Old Symbol |  |

| COMPRESSED GAS | What does it mean? Contains gas under pressure; may explode if heated. Contains refrigerated gas; may cause cryogenic burns or injury. Warning label examples Protect from sunlight Wear cold insulating gloves/face shield/eye protection. Get immediate medical advice/attention. Examples of where you would find it Gas containers |
| --- | --- |
| Old Symbols | no existing symbol |

| CORROSIVE | What does it mean? May be corrosive to metals Causes severe skin burns and eye damage Warning label examples Do not breathe dust/fume/gas/mist/vapours/spray Wash…thoroughly after handling Wear protective gloves/protective clothing/eye protection/face protection Store locked up Keep only in original container Examples of where we can find it Drain cleaners, acetic acid, hydrochloric acid, ammoniac |
| --- | --- |
| Old Symbols |  |

## Health hazards pictograms

| TOXIC | What does it mean? Fatal if swallowed Fatal in contact with skin Fatal if inhaled Toxic: if swallowed Toxic in contact with skin Toxic if inhaled Warning label examples Wash... thoroughly after handling. Do not eat, drink or smoke when using this product. If swallowed: immediately call a POISON CENTER or a doctor/physician Rinse mouth Store in a closed container Do not get in eyes, on skin, or on clothing. Wear protective gloves/protective clothing/eye protection/face protection. If on skin: gently wash with plenty of soap and water Remove/take off immediately all contaminated clothing. Wash contaminated clothing before reuse. Do not breathe dust/fume/gas/mist/vapours/spray. Use only outdoors or in a well-ventilated area Wear respiratory protection If inhaled: Remove victim to fresh air and keep at rest in a position comfortable for breathing Store locked up Examples of where we can find it Pesticide, biocide, methanol |
| --- | --- |
| Old Symbols |  |

| IRRITANT | What does it mean? May cause respiratory irritation May cause drowsiness or dizziness May cause an allergic skin reaction Causes serious eye irritation Causes skin irritation Harmful if swallowed Harmful in contact with skin Harmful if inhaled Harms public health and the environment by destroying ozone in the upper atmosphere Warning label examples Avoid breathing dust/fume/gas/mist/vapours/spray Use only outdoors or in a well-ventilated area If inhaled: remove victim to fresh air and keep at rest in a position comfortable for breathing If swallowed: call a POISON CENTER or a doctor/physician if you feel unwell Wear protective gloves/protective clothing/eye protection/face protection. If on skin: wash with plenty of soap and water If in eyes: rinse cautiously with water for several minutes. Remove contact lens, if present and easy to do. Continue rinsing. Do not eat, drink or smoke when using this product. Examples of where we can find it Washing detergents, toilet cleaner, coolant fluid |
| --- | --- |
| Old Symbol |  |

| HEALTH HAZARD | What does it mean? May be fatal if swallowed and enters airways Causes damage to organs May cause damage to organs May damage fertility or the unborn child Suspected of damaging fertility or the unborn child May cause cancer Suspected of causing cancer May cause genetic defects Suspected of causing genetic defects May cause allergy or asthma symptoms or breathing difficulties if inhaled Warning label examples If swallowed: immediately call a POISON CENTER or a doctor/physician Do NOT induce vomiting Store locked up Do not breathe dust/fume/gas/mist/vapours/spray. Wash thoroughly after handling. Do not eat, drink or smoke when using this product. Get medical advice/attention if you feel unwell If exposed: Call a POISON CENTER or doctor/physician Obtain special instructions before use Do not handle until all safety precautions have been read and understood Use personal protective equipment as required If exposed or concerned: Get medical advice/attention Avoid breathing dust/fume/gas/mist/vapours/spray In case of inadequate ventilation wear respiratory protection If inhaled: If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing Examples of where we can find it? Turpentine, petrol, lamp oil |
| --- | --- |
| Old Symbols |  |

## Environmental hazards pictograms

| ENVIRONMENTALLY DAMAGING | What does it mean? Very toxic to aquatic life with long lasting effects Toxic to aquatic life with long lasting effects Warning label examples Avoid release to the environment Collect spillage Examples of where we can find it? Pesticides, biocides, petrol, turpentine |
| --- | --- |
| Old Symbol |  |

Please feel free to watch the following information videos featuring “Napo”. Each film is co-produced by a number of European Institutions. The European Agency for Safety and Health at Work in Bilbao, Spain has funded the development of the website.

http://www.napofilm.net/en/napos-films/multimedia-film-episodes-listing-view?filmid=napo-012-danger-chemicals

http://www.napofilm.net/en/napos-films/multimedia-film-episodes-listing-view?filmid=napo-013-protect-your-skin

# Safety Labels

In addition to the pictograms found on chemical containers, the label also has an explanation of what it means, called a hazard statement. One pictogram might have multiple hazard statements about the effects that the substance might cause. For example, the exclamation mark symbol might mean that the substance may cause skin irritation or that it may cause serious eye irritation or respiratory irritation.

You will also find precautionary statements on these labels, which explain how best to handle the product safely and what you should do if you are exposed to the product by accident. The label also indicates the severity of the damage the product may cause: ‘Danger' for more severe damage and ‘Warning' for less severe.

Always make sure you know what the labels mean and read the instructions before you use the product. If the label is missing or is damaged to the point you can no longer read it, do not use it.

A hazard label must contain the following elements applicable to the substance or mixture placed on the market as shown in figure 1.

- name, address and telephone number of the EU supplier(s),

- product identifiers e.g. chemical name and CAS/EC no. of the substance/trade name of a mixture along with the chemical name(s) of all substances responsible for classification of the mixture (excluding skin and eye irritants),

- hazard pictogram(s):signal word: either Warning or Danger depending on the classification

- hazard statement(s) description of the hazardous effect e.g. Harmful if inhaled,

- the appropriate precautionary statements to allow the user of the chemical to take measures to protect health/environment e.g. Keep out of reach of children,

- nominal quantity (when the chemical is supplied to the general public), and

- supplemental information.

## Protecting others and the environment

### What you need to do

The law[1] says you must keep every part of your construction site in ‘good order’ and every place of work clean. The objective is to achieve what is usually called a good standard of ‘housekeeping’ across the site.

All contractors should plan, manage and monitor their work so it is carried safely and without risks to health. This includes careful planning on how the site will be kept tidy and housekeeping is actively managed.

Each year around 1000’s trips or slips on construction sites result is minor accidents to fatalities. These incidents can cause huge impacts on both work and personal life. Many are caused simply because there is something in the person’s way, such as building materials or waste.

### Materials storage

Safe and efficient materials storage depends on good co-operation and co-ordination between everyone involved including, client, contractors and suppliers.

On all projects the arrangements for materials storage should be discussed and agreed between contractors and the project client. Larger projects should have arrangements for materials storage included in the planning stage.

Key things to consider:

- Storage areas - designate storage areas for plant, materials, waste, flammable substances e.g. foam plastics, flammable liquids and gases such as propane and hazardous substances e.g. pesticides and timber treatment chemicals;

- Flammable materials - will usually need to be stored away from other materials and protected from accidental ignition;

- Pedestrian routes – do not allow storage to ‘spread’ in an uncontrolled manner on to footpaths and other walkways. Do not store materials where they obstruct access routes or where they could interfere with emergency escape;

- Storage at height - if materials are stored at height e.g. on top of a container, make sure necessary guard rails are in place if people could fall when stacking or collecting materials or equipment;

- Keep tidy - keep all storage areas tidy, whether in the main compound or on the site itself.

## Effects on the environment

The construction industry is a major source of pollution, responsible for more water pollution incidents than any other industry, and thousands of noise complaints every year. Although construction activities also pollute the soil, the main areas of concern are: air, water and noise pollution.

#### Noise Pollution

Construction sites produce a lot of noise, mainly from vehicles, heavy equipment and machinery, but also from people shouting and radios turned up too loud. Excessive noise is not only annoying and distracting, but can lead to hearing loss, high blood pressure, sleep disturbance and extreme stress. Research has shown that high noise levels disturb the natural cycles of animals and reduces their usable habitat.

#### Air Pollution

Construction activities that contribute to air pollution include: land clearing, operation of diesel engines, demolition, burning, and working with toxic materials. All construction sites generate high levels of dust (typically from concrete, cement, wood, stone, silica) and this can carry for large distances over a long period of time. Construction dust is often invisible to the naked eye.

Research has shown that dust penetrates deeply into the lungs and causes a wide range of health problems including respiratory illness, asthma, bronchitis and even cancer. Another major source of air pollution on construction sites comes from the diesel engine exhausts of vehicles and heavy equipment. This is known as diesel particulate matter (DPM) and consists of soot, sulphates and silicates, all of which readily combine with other toxins in the atmosphere, increasing the health risks of particle inhalation.

Diesel is also responsible for emissions of carbon monoxide, hydrocarbons, nitrogen oxides and carbon dioxide. Noxious vapours from oils, glues, thinners, paints, treated woods, plastics, cleaners and other hazardous chemicals that are widely used on construction sites, also contribute to air pollution.

#### Water Pollution

Sources of water pollution on building sites include: diesel and oil; paint, solvents, cleaners and other harmful chemicals; and construction debris and dirt. When land is cleared it causes soil erosion that leads to silt-bearing run-off and sediment pollution. Silt and soil that runs into natural waterways turns them restricts sunlight filtration and destroys aquatic life.

Surface water run-off also carries other pollutants from the site, such as diesel and oil, toxic chemicals, and building materials like cement. When these substances get into waterways they poison water life and any animal that drinks from them. Pollutants on construction sites can also soak into the groundwater, a source of human drinking water. Once contaminated, groundwater is much more difficult to treat than surface water.

## Measures to Prevent Pollution

Good construction site practice can help to control and prevent pollution. The first step is to prepare environmental risk assessments for all construction activities and materials likely to cause pollution. Specific measures can then be taken to mitigate these risks:

- To prevent erosion and run-off, minimise land disturbance and leave maximum vegetation cover.

- Control dust through fine water sprays used to dampen down the site.

- Screen the whole site to stop dust spreading, or alternatively, place fine mesh screening close to the dust source. (As demonstrated in the picture below)

- Cover skips and trucks loaded with construction materials and continually damp down with low levels of water.

- Cover piles of building materials like cement, sand and other powders, regularly inspect for spillages, and locate them where they will not be washed into waterways or drainage areas.

- Use non-toxic paints, solvents and other hazardous materials wherever possible

- Segregate, tightly cover and monitor toxic substances to prevent spills and possible site contamination.

- Cover up and protect all drains on site.

- Collect any wastewater generated from site activities in settlement tanks, screen, discharge the clean water, and dispose of remaining sludge according to environmental regulations.

- Use low sulphur diesel oil in all vehicle and equipment engines, and incorporate the latest specifications of particulate filters and catalytic converters.

- No burning of materials on site.

- Reduce noise pollution through careful handling of materials; modern, quiet power tools, equipment and generators; low impact technologies; and wall structures as sound shields.