MODULE 2

HAZARDOUS AND HARMFUL FACTORS IN CONSTRUCTION

Project: 2013-1-BG1-LEO05-088722

Author:	Assist. Prof. Dr. Petar Antov, Assist. Prof. Dr. Tanya Pencheva

University of Forestry, Bulgaria

October, 2014

This project has been funded with support from the European Commission. This publication reflects the views only of the author, and the Commission cannot be held responsible for any use which may be made of the information contained therein.

Table of Contents

OVERVIEW	2

1. Working at height	3

2. Slips, trips and falls	11

3. Vehicle accidents	11

4. Lifting operations	14

5. Manual handling and musculoskeletal disorders (MSDs)	16

6. Electricity	18

7. Fire	20

8. Demolition works	21

9. Dangerous substances	22

10. Noise	23

11. Vibration	24

12. Work-related stress	25

References	26

# OVERVIEW

In the European Union construction is one of the most physically demanding sectors and the sector most at risk of accidents, with more than 1300 people being killed in occupational accidents every year. Around 45% of construction workers say their work affects their health.. The costs of these accidents are immense to the individual, to the employer and to society and they can amount to an appreciable proportion of the contract price. Over 99% of construction firms in Europe are small and medium-sized enterprises (SMEs). SMEs are therefore most affected by construction accidents.

Workers in the construction sector have greater exposure to biological, chemical and ergonomic hazardous and harmful factors. The main hazards include working at height; slips, trips and falls; vehicle accidents; lifting operations; manual handling and musculoskeletal disorders (MSDs); electricity; fire; demolition works; exposure to loud noise, vibration, work-related stress etc. In the course of their work, construction workers may come into contact with many dangerous substances which can lead to serious health problems and occupational illnesses. The hazards are not, however, restricted to those working on sites. People are often killed or injured because construction activities have not been adequately controlled.

Priority must be given to preventive measures that eliminate or reduce the hazard at source and provide collective protection. Individual protection, such as personal protective equipment, should be used where risks can’t be further reduced by other means. Ongoing monitoring and regular inspections are needed in addition to the overall risk assessment.

The safety and health of tomorrow’s workforce is dependent on the integration of occupational safety and health into education today. Children and young people need to learn about safety and health at an early stage of their education so that they can carry these ideas forward into their future working and private lives. The importance of the integration of OSH training into all levels of education, including vocational and higher education, has been recognized as one of the key factors to strengthen the prevention culture. Education about health and safety does not start with entry into the world of work, it should be part of the school curriculum or a vocational subject in its own right.

The sooner existing and future professionals in the field of construction get acquainted with the concept of safety and health, the sooner they can develop OSH awareness, and the better they can shape their own safety and health environment in their working and private lives.

# 1. Working at height

Falls from heights are the most common cause of injury and death. Causes include: working on a scaffold or platform without guard rails or without a safety harness correctly attached; fragile roofs; and ladders that are badly maintained, positioned and secured.

Falls are the largest cause of accidental death in the construction industry. They account for 50% of all fatalities. There is no distinction between low and high falls. This means that for any work at height, precautions are required to prevent or minimise the risk of injury from a fall.

The main principles when working at height should be:

- avoiding working at height where possible;

- using work equipment to prevent falls where work at height cannot be avoided;

- where the risk of a fall cannot be eliminated, work equipment should be used to minimise the distance and consequences of falls;

- collective protection measures (scaffolds, nets, soft landing systems) should be taken in order to protect those at risk;

- work should be carried out only when weather conditions don’t threaten the health and safety of construction workers.

When planning for working at height that is unavoidable, the first choice will be to use any existing safe place of work that allows safe access and provides a safe working place. Where it is not reasonably practicable to work safely from the existing place of work, an alternative means of access will be needed. This will involve the use of work equipment.

Traditionally, much work has been done from scaffolding. However, other means of access (such as mobile elevating work platforms (MEWPs) and tower scaffolds) will ensure collective fall prevention because they are equipped with guard rails. Personal measures, such as podium steps, can also be used to prevent falls. If fall prevention is not reasonably practicable, other work equipment can be used to minimise the distance and consequences of a fall using, for example, personal suspension equipment such as rope access techniques and boatswain’s chairs.

Ladders are at the bottom of the hierarchy because they do not prevent or mitigate a fall. However, if they are used by competent people and are regularly inspected and well maintained, then their use may be justified providing it is not reasonably practicable to use other work equipment which will prevent or mitigate a fall.

Safe working platforms

A working platform is virtually any surface from which work is carried out, such as a roof, scaffold, MEWP, tower scaffold, trestle etc. It becomes a safe working platform when you can’t fall off it or through it or when measures have been taken to mitigate a fall from it.  Working platforms should be: capable of supporting the weight of the workers, any materials and equipment; stable and will not overturn; wide enough to allow people to pass back and forth safely and use any equipment or material necessary for their work at that place; kept clear of loose materials and constructed to prevent materials from falling; free of openings and traps through which people’s feet could pass, causing them to trip, fall or be injured in any other way; kept free of tripping and slipping hazards. The platforms should be kept clean and tidy.

General access scaffolds

Suitable precautions must be taken to prevent falls. General access scaffolds provide a means of working at height while preventing falls and should be provided whenever practicable.

Scaffolds should be designed, erected, altered and dismantled only by competent people and the work should always be carried out under the direction of a competent supervisor. Scaffolders should always adopt methods of work to prevent falls during the erection of scaffolding. This can be achieved by using an advanced guard rail system. Where this is not practicable, harnesses should be worn to provide a means of fall arrest.

Scaffolds should never be erected or dismantled over people or busy pavements. If the work presents a danger to the public, a road closure should be applied to eliminate the risk of injuries.

The scaffold should be based on a firm, level foundation. The ground or foundation should be capable of supporting the weight of the scaffold and any loads likely to be placed on it. The scaffolding should be braced and tied into a permanent structure or otherwise stabilised (Figure 1). Rakers only provide stability when they are braced and footed adequately; single-tube rakers alone do not usually provide this and need to be braced to prevent buckling.

Figure 1. An independent scaffold

Guard rails, toe boards and brick guards

Guard rails, toe boards and other similar barriers should be provided to prevent falls whenever practicable.

Figure 2. Guard rails and toe boards

They should be: strong and rigid enough to prevent people from falling and be able to withstand other loads likely to be placed on them; fixed to a structure or part of a structure capable of supporting them; include a main guard rail, a toe board and brick guards, a sufficient number of intermediate guard rails.

Tower scaffolds

Tower scaffolds (also known as mobile access scaffolds) (Figure 3) are widely used and can provide an effective and safe means of gaining access to work at height while preventing falls. Inappropriate erection and misuse of tower scaffolds are the cause of numerous accidents each year. Aluminium towers are light and can easily overturn if used incorrectly. Towers rely on all parts being in place to ensure adequate strength. They can collapse if sections are left out.

Figure 3. Tower scaffold

Mobile access equipment

Where it is not possible to avoid work at height or work from an existing structure, mobile access equipment, including mobile elevating work platforms (MEWPs) and mast climbing work platforms (MCWPs), can be used to prevent falls.

MEWPs can provide excellent safe access to high-level work that can be easily moved from one location to another. The three basic types of MEWP are scissor lift (Figure 4) (which gives a vertical lift only), telescopic boom (which gives vertical lift and outreach), articulating and telescopic boom (usually vehicle mounted and used for special activities such as bridge repairs).

Figure 4. Scissor lift

MCWPs are often used when carrying out repairs to (or refurbishment of) high-rise buildings. MCWPs are designed to provide access to working positions – they are not designed to act as material hoists. A great advantage of using MCWPs is that workers can be protected from adverse weather as many types can be provided with screens and a roof to the platform. Enclosures to platforms can increase wind loads so the supplier must always be consulted before fixing them.

Figure 5. Mast climbing work platform (MCWP)

Suspended access equipment

Where it is not possible to use mobile access equipment, suspended access equipment can be used. This enables the worker place themselves in the position required to carry out the work and includes suspended access cradles and boatswain’s chairs.

Safety nets and soft landing systems

Safety nets and soft landing systems (e.g. fire-retarded polystyrene-filled bags, air bags etc) can be used as leading edge protection to mitigate the consequences and distance should a fall occur. They are not a substitute for the use of fall prevention measures but can be used in conjunction with them if the risk of a fall cannot be eliminated (see Figure 15).

Safety harnesses

Fall-protection equipment can prevent people falling when it is used as a work-restraint system. When used in this way, the lanyard is kept as short as possible while allowing operators to reach their place of work. This prevents them from getting into a fall position, as they are physically unable to get close enough to the open edge (see Figure 6). Using a harness in this way is acceptable for light short-duration work and for inspection and maintenance purposes. In some cases a permanent or temporary horizontal lifeline could allow safe working. A harness can also be used as a work restraint when working from MEWPs or cradles.

Figure 6. Safety harness

Ladders and stepladders

Ladders and stepladders are among the most commonly used pieces of access equipment on site and perhaps the most misused. They should be used for short-duration work (15-30 minutes in one position depending on the risk assessment); for light work (they are not suitable for strenuous tasks which may involve carrying materials or supporting components); if a secure handhold is available.

Roof work and fragile surfaces

Almost one in five workers killed in construction accidents are doing roof work. Some are specialist roofers, but many are simply involved in repairing and cleaning roofs. The main causes of accidents are falling off the edges of roofs and falling through holes, roof lights and other fragile surfaces.

Compliance with well-established safety procedures and existing legislation could save lives and prevent injuries. All roof work requires a risk assessment and, if the work is extensive, a method statement that sets out a safe system of work. Most accidents could be avoided if the most suitable equipment was used. Independent scaffolds that provide safe access onto the roof, a safe working platform and the capacity for material storage are the ideal solution.

On fragile roofs, the work has to be carefully planned to prevent falls through the roof. All work should be carried out from beneath where practicable. Where this is not possible, the use of MEWPs allows the workers to carry out the work from within the MEWP basket without standing on the roof itself.

If access onto a fragile roof cannot be avoided, edge protection should be installed around the perimeter of the roof and staging should be used to spread the load.

Roof openings and fragile roof lights are a particular hazard. Some roof lights are difficult to see in certain light conditions and others may be hidden by paint. Protection from falling through openings and fragile roof lights must be provided using either barriers or covers that are secured or labelled with a warning. If the work is the replacement of roof lights, nets slung close beneath the roof lights, or a harness attached to a work positioning line which is fixed to the staging, provides protection should a fall occur.

Formwork and reinforced concrete structures

The main risks are: people falling during erection and striking of formwork and assembly of the steel frame; collapse of the formwork; materials falling while striking the formwork; manual handling of shutters, reinforcing bars etc.; being struck by the concrete skip; silica dust and hand-arm vibration from scabbling operations; awkward postures and working positions for steel fixers; dermatitis and cement burns from wet concrete.

Many of these risks can be reduced or removed by design and careful planning: designers should consider the manual handling risks when detailing size and length of the reinforcing bar; fixing reinforcement steel in prefabricated sections in factory conditions and craning it into position so that work can be done on benches to reduce the need for bending down; using formwork systems that have edge protection and access designed in; minimising the need for scabbling by using retarders; and using concrete pumps instead of cranes and skips.

Many formwork systems have purpose-designed fittings to allow access platforms to be fitted and they should be used (Figure 7a).

a)                                                                 b)

Figure 7. a) a formwork system with purpose-designed fittings; b) a tower scaffold

Climbing up vertical sections of reinforcement or up the outside of column formwork is not permitted. A tower scaffold can provide safe access to columns (Figure 7b).

# 2. Slips, trips and falls

Slips or trips represent the largest cause of injuries on construction sites, reported each year. Most of these injuries can be easily avoided by effective management of those areas of the site where workers need access, such as corridors, footpaths, stairwells and site cabins.

The main causes of this type of accident are: having to walk over uneven ground, particularly when carrying unwieldy objects; tripping over building materials or waste lying around; tripping over trailing cables; slipping caused by wet surfaces or poor ground conditions; trips caused by small changes in level. Although each of the problems can be dealt with quite simply, in practice it is difficult because site conditions are constantly changing. It is therefore essential that site managers exercise good control and that everybody working on site takes responsibility for ensuring that the way they do their work does not create a risk for others.

The following measures should be taken in order to prevent accidents: work and storage areas should be kept tidy; all corridors, stairways, footpaths and other areas, used by pedestrians, should be kept clear of obstructions; sound temporary ramps should be used in order to provide easy and safe access in case of small changes of level; the disposal of waste materials should be properly arranged; the inside and outside work areas should be adequately lit; workers should wear footwear providing good grip; cordless tools should be used whenever possible; steps, leading to site cabins should be properly constructed.

# 3. Vehicle accidents

Every year, workers are killed on construction sites by moving vehicles, fall from vehicles, are struck by objects falling from vehicles, or are injured by vehicles overturning. Many more are seriously injured in this way. The risks can be reduced if the use of vehicles and mobile plant is properly managed.

The main protective measures include: separation of vehicle and pedestrian routes, putting safety signs and signboards to warn of the hazards, checking reversing lights and sounds of construction vehicles, moving routes away from vulnerable or potentially hazardous structures, providing training to ensure workers drive safely.

Gates or barriers to control entry into the work area should be used. The location of stores/goods receiving areas should be planned carefully in order to reduce the need for delivery vehicles to travel through construction site.

Appropriate speed limits for the construction site routes should be set. The limits should be clearly displayed using signs and signboards.

The need for vehicles to reverse should be avoided by drive-in/drive-out access to delivery and work areas. When roads are narrow or constricted by park vehicles or stored materials, a one-way system may be considered. All turning areas should be fenced off in order to exclude pedestrians, and they should be kept free from obstructions and parked vehicles.

Where walkways need to cross vehicle routes, a clearly signed, well lit crossing point should be provided. Both drivers and pedestrians should be able to clearly easily see each other as they approach the crossing (Figure 8).

Figure 8. Provision of a clearly signed, well-lit crossing point

All drivers and pedestrians should know and understand the routes and traffic rules on the site and induction training for drivers, workers and visitors should be provided. Plans showing the traffic routes at site entrances should be posted. Standard road signs should be provided in order to warn, guide and instruct drivers on site. In particular, the routes for delivery drivers and site visitors should be clearly signed.

Any temporary structures, such as scaffolds, which might be damaged or made unsafe if struck by a vehicle, should be protected. Where vehicles tip materials into excavations, it is necessary to take preventive measures, such as stop blocks (Figure 9). The vehicles should not be overloaded, as it may obstruct the driver’s view and they may become unstable, difficult to steer or the brakes may be inadequate to stop the vehicle.

Figure 9. Stop blocks

For any particular task there is often more than one item of plant or type of vehicle that could undertake it. Selecting the right plant or vehicle can make the job significantly safer. Many construction vehicles have significant blind spots (areas that the driver cannot see) in various locations around the vehicle. In some cases this can create risks to both nearby pedestrians and to the vehicle when the vehicle is moving. Any load that the vehicle is carrying can create temporary blind spots. Some vehicles are fitted with vision aids such as mirrors or CCTV, which help drivers to see areas that they cannot view directly. If these aids are available, it should be checked regularly that they are working, properly adjusted and that the driver has been instructed in their use (Figure 10).

Figure 10. Examples of vision aids

Signallers (with a protected position from which they can work safely) should be provided wherever possible to direct vehicle movements. Signallers should be easily distinguished on site, e.g. by providing them with distinctive clothing. Drivers under the control of signallers should be instructed that if they cannot see the signallers they should stop immediately.

# 4. Lifting operations

Many construction workers are killed or seriously injured during lifting operations because of accidents such as: cranes overturning; material falling from hoists; and slinging failures.

To avoid the risk of injury it is essential that all material handling is properly planned. Where possible, avoid people having to lift materials at all. Where lifting is unavoidable mechanical handling aids should be provided. All equipment used for lifting should be in good condition and should be used by trained and competent workers.

Hoists

Hoists of various types are widely used on construction sites and include goods and passenger hoists, inclined hoists and transport platforms (Figure 11). They have become an essential part of almost any construction project providing the facility for the smooth and rapid movement of workers and materials. Hoists must be: properly constructed of sound materials and capable of lifting the required loads; properly marked as to use either for equipment and materials only, or for passengers in addition to goods, and the number that can be carried, together with a safe working load notice (passengers should not be allowed to ride on a goods-only hoist); operated only by trained and competent people; thoroughly examined and tested after erection, substantial alteration or repair and at relevant intervals.

Figure 11. Construction hoist

Regular checks should be carried out and the results recorded. As a general guide, weekly checks should suffice.

Cranes

All construction jobs are likely to involve lifting operations, even if it is only the use of a lorry loader to deliver materials to site. Lifting operations can be hazardous activities if they are not properly planned and carried out safely. Cranes provide a versatile, reliable means of lifting on site. However, it is easy to become complacent about their safe use and complacency can lead to serious accidents.

Safe lifting depends on three key elements, which require that the operation must be:

properly planned by a competent person; appropriately supervised (by a competent ‘crane supervisor’); and carried out in a safe manner.

Planning for a safe lifting operation includes: selecting the right crane for the job, able to lift the heaviest load at the required radius with capacity to spare; positioning the crane in a safe place, so that the crane operator has a clear view, away from overhead power lines, excavations and railway lines; it is on level ground that can take its full weight and its load (timber packing may be needed); making sure a rated capacity indicator (automatic safe load indicator) is fitted (when the crane is able to lift more than one tonne) and is in good working order; selecting the most suitable lifting accessories (e.g. chains, slings, wire rope, hooks, etc.) and method of slinging (choker sling, double wrap, cradle sling, etc.) relative to the weight, shape, centre of gravity, lifting points and load stability; checking that the crane and any lifting accessories have a current thorough examination report; making sure the crane supervisor, crane operator, slinger and signaller are trained.

The following rules should be taken into consideration when carrying out lifting operation: load routes should be established to avoid loads being lifted over people; the load should be properly slung by a competent person; chains and slings should be of the correct strength and in good condition; loads should be slung so that they are in balance with the centre of gravity beneath the hook (Figure 12); there should be adequate clearance so that people are not struck or trapped by the load, counterweight or body of the crane.

Figure 12. Examples of stable and unstable lifting of loads

# 5. Manual handling and musculoskeletal disorders (MSDs)

Lifting and moving loads manually are one of the most common causes of injury at work. Many manual handling injuries result from repeated operations, but even one bad lift can cause a lifetime of pain and disability. Employers should avoid the need to carry out manual handling which creates a risk of injury. Where avoidance is not reasonably practicable, employers have to make an assessment, reduce the risk of injury as far as reasonably practicable and provide information about the weight of loads.

There is no truly ‘safe’ weight limit for manual handling operations. The degree of risk associated with lifting varies according to the nature of the load, the circumstances in which the lift takes place, how often the lifting operation is carried out and the weight of the item that is being lifted.

When manual handling is necessary, injuries should be prevented by: avoiding unnecessary handling; identification of operations which involve either lifting heavy or awkward loads or repetitive lifting operations (it is essential to find out the weight of heavy items which may have to be manually handled); positioning loads by machine and planning to reduce the height from which they have to be lifted and the distance over which they have to be carried; setting limits on the size of commonly used products or material; ordering bagged materials in small, easily handled sizes where possible; training workers in safe lifting techniques and sensible handling of loads.

Gin wheels (Figure 13) provide a convenient way of raising tools and light loads manually. Though simple pieces of equipment, care is needed when assembling and using them if accidents are to be avoided. If a gin wheel or similar is to be used, it should be securely fixed to a secure anchorage to prevent displacement; a proper hook designed to prevent load displacement or a hook fitted with a safety catch should be used.

Figure 13. Gin wheel

Musculoskeletal disorders (MSDs)

Musculoskeletal disorders (MSDs) are the most common work-related problem in Europe.  Work-related musculoskeletal disorders are impairments of bodily structures such as muscles, joints, tendons, ligaments, nerves, bones and the localised blood circulation system, that are caused or aggravated primarily by work and by the effects of the immediate environment in which work is carried out.

Most work-related MSDs are cumulative disorders, resulting from repeated exposure to high or low intensity loads over a long period of time. However, MSDs can also be acute traumas, such as fractures, that occur during an accident. These disorders mainly affect the back, neck, shoulders and upper limbs, but can also affect the lower limbs.

MSDs are prevalent across a range of trades and particularly among bricklayers, plasterers and joiners. Where the risk of MSDs cannot be removed at the planning phase, employers should carry out a risk assessment to identify the hazards, assess the risks, and take action to prevent MSDs developing. If possible, manual handling should be eliminated by, for example, using cranes, hoists, vacuum-lifting devices or conveyor systems.

# 6. Electricity

Electrical equipment is used on virtually every construction site. Everyone is familiar with it, but unlike most other hazards, which can be seen, felt or heard, there is no advance warning of danger from electricity, and electricity can kill.

Electrical systems and equipment must be properly selected, installed, used and maintained. Hazards arise through faulty installations, lack of maintenance and abuse of equipment. Accidents happen because people are working on or close to electrical equipment; workers have not received adequate training or adequate precautions have not been taken.

Electrical equipment used on building sites (particularly power tools and other portable equipment and their leads) faces harsh conditions and rough use. It is likely to be damaged and therefore become dangerous. Modern double-insulated tools are well protected, but their leads are still vulnerable to damage and should be regularly checked.

If mains voltage has to be used, the risk of injury is high if equipment, tools, leads etc are damaged, or there is a fault. Residual current devices (RCDs) with a rated tripping current not greater than 30 mA with no time delay will be needed to ensure that the current is promptly cut off if contact is made with any live part. RCDs must be installed and treated with great care if they are to save life in an accident. They have to be kept free of moisture and dirt and protected against vibration and mechanical damage. They need to be properly installed and enclosed, including sealing of all cable entries. They should be checked daily by operating the test button.

Tools and equipment should be suitable for site conditions. DIY tools and domestic plugs and cables are not designed to stand up to everyday construction work.

If work is to be done in areas where there is a risk of flammable vapours (such as in a petrochemical works), it will be necessary to select specially designed electrical equipment to prevent it acting as a source of ignition due to sparks and overheating.

Overhead power lines

Contact with overhead power lines is a regular cause of death and injury. Any work near electric distribution cables or railway power lines must be carefully planned to avoid accidental contact.

The most common operations leading to contact with overhead lines are operating cranes and other lifting equipment; raising the body or inclined container of tipper lorries; operating excavators and other earth-moving equipment; handling long items such as scaffold tubes, metal roof sheets, ladders, etc.; using MEWPs.

Where possible all work likely to lead to contact with the overhead line should be done in an area well clear of the line itself. Where this is not feasible, either the power line must be made dead or suitable precautions must be taken to prevent any danger. For some jobs, it may be necessary for the electricity supplier to isolate or re-route overhead lines to enable work to proceed.

In cases where approach is likely, stout, distinctive barriers should be erected at ground level to prevent access (Figure 14). Where work is to take place close to overhead lines, detailed precautions should be discussed with the owner of the lines. However, the responsibility for ensuring that precautions are adequate remains with the contractors undertaking the work, not with the owner of the power lines.

Figure 14. Ground-level barrier for plant working near an overhead power line

# 7. Fire

Many solids, liquids and gases can catch fire and burn. It only takes a source of ignition, which may be a small flame or an electrical spark, together with air. Any outbreak of fire threatens the health and safety of those on site and will be costly in damage and delay. It can also be a hazard to people in surrounding properties. Fire can be a particular hazard in refurbishment work when there is a lot of dry timber and at the later stages of building jobs where flammable materials such as adhesives, insulating materials and soft furnishings are present.

Many fires can be avoided by careful planning and control of work activities. Good housekeeping and site tidiness are important not only to prevent fire, but also to ensure that emergency routes do not become obstructed. Making site rules can help.

The following precautions should be taken to prevent fires: less-easily ignited and fewer flammable materials, e.g. use water-based or low-solvent adhesives and paint should be used; the quantity of flammables at the workplace should be kept to a minimum; flammable liquids should always be kept in suitable closed containers; if work involving the use of flammable materials is being carried out, smoking should be prohibited; pipes, barrels, tanks etc. which may have contained flammable gases or liquids should be purged or otherwise made safe before using hot cutting equipment; to minimise the risk of gas leaks and fires involving gas-fired plant valves on gas cylinders should be closed when not in use; hoses should be regularly checked for wear and leaks; oil or grease coming into contact with oxygen cylinder valves should be avoided; flammable solids, liquids and gases should be stored safely; there should be an extinguisher to hand when doing hot work such as welding or using a disc cutter that produces sparks; highly flammable waste such as solvent-soaked rags should be collected separately in closed fire-resisting containers.

A system to alert people on site should be set; this could be a temporary or permanent mains operated fire alarm (which should be tested regularly, eg weekly), a klaxon, an air horn or a whistle, depending on the size and complexity of the site. Any warning needs to be distinctive, audible above other noise and recognisable by everyone.

Escape routes should be planned and remain available and obstructed at any time. Well separated alternative ways should be provided for work areas above or below ground. Emergency escape signs are needed so that people will know the escape routes (see Figure 15).

Figure 15. Emergency exit sign

As well as providing fire extinguishers for hot work, fire extinguishers should be located at identified fire points around the site. The extinguishers should be appropriate to the nature of the potential fire:

- wood, paper and cloth – water extinguisher;

- flammable liquids – dry powder or foam extinguisher;

- electrical – carbon dioxide (CO2) extinguisher.

# 8. Demolition works

Demolition, dismantling and structural alteration are high-risk activities whose safe execution is complex and technical and where expertise is vital. They require careful planning and execution by contractors who are competent in the full range of demolition techniques.

During demolition and dismantling, workers are injured in falls from edges and through openings and fragile surfaces. Workers and passers-by can be injured by the premature and uncontrolled collapse of structures or parts of structures and by flying debris. Effective traffic management systems are essential to avoid workers being put at risk of being hit by slewing or reversing plant. Dust, noise and vibration are also significant problems that need to be considered and controlled when planning any demolition work

It is essential that demolition is planned and carried out by trained demolition workers under the supervision of a competent person. Supervisors should have knowledge of the particular type of demolition being carried out, its hazards and how to control them. In particular, they should understand and follow any demolition method statement and know of any particular demolition sequence required to avoid accidental collapse of the structure.

Before work starts, the site should be surveyed for hazardous material and structural condition.

# 9. Dangerous substances

The construction industry has a poor health record. Construction workers are likely to suffer ill health as a result of their work in the industry after exposure to both harsh working conditions and hazardous substances. Harm could be caused by:

•	Breathing in fumes, vapours, dust;

•	Direct contact with skin;

•	Swallowing or eating contaminated material.

In the course of their work, construction workers may come into contact with many dangerous substances. The most common types include:

- cement, lime and plaster;

- paint;

- tar, pitch and bitumen;

- solvents, thinners and de-greasers;

- certain epoxy, acrylic and formaldehyde resins;

- asbestos;

- mineral oils and grease.

Any hazardous substances that are going to be used, or processes which may produce hazardous materials, should be identified. The risks from work that might affect site workers or members of the public should then be assessed. Designers should eliminate hazardous materials from their designs. Where this is not possible, they should specify the least hazardous products which perform satisfactorily.

People may be exposed to hazardous substances either because they handle or use them directly (eg cement and solvents), or because the work itself results in the creation of a hazardous substance (e.g. scabbling concrete generates silica dust). Both kinds of hazard should be identified and assessed.

If hazardous substances are going to be used, manufacturers and suppliers of such substances have a legal duty to provide information (Material Safety Data Sheets).

Some hazardous substances may be on site before any work starts, e.g. sewer gases or ground contaminants. These risks should be assessed in the same way as for other hazardous substances. Information to help identify these risks may be available from the client, the design team or the principal contractor and should be contained in the pre-construction-stage health and safety plan.

Storage areas for hazardous substances should be designated. Flammable materials usually need to be stored away from other materials and protected from accidental ignition.

Substances can also be a hazard to health when they are transferred from workers’ hands onto food, cigarettes etc. and so taken into the body. This can be avoided by good personal hygiene, eg by washing hands and face before eating, drinking and smoking; eating, drinking and smoking only away from the work area. It is essential to provide washing facilities, with a supply of hot and cold (or warm) running water, soap and a means of drying the hands.

All construction workers are potentially at risk from exposure to asbestos. Asbestos fibres were used for many years in thermal insulating materials such as lagging and coatings, ‘fireproof’ textiles, paper and boards, asbestos cement products, electrical insulating materials and personal protective equipment. Asbestos fibres can have serious health effects if inhaled, including asbestosis, lung cancer and mesothelioma.

There is no known safe exposure level to asbestos. That is why, if there is a suspicion there may be asbestos present, the work should be stopped and only properly trained workers should deal with it.

Several illnesses have been linked to the construction sector, among them: silicosis among sand blasters, tunnel builders and rock drill operators; asbestosis (and other diseases caused by asbestos) among asbestos insulation workers, steam pipe fitters, building demolition workers and others; bronchitis among welders; skin allergies among masons and others who work with cement; neurologic disorders among painters and others exposed to organic solvents and lead.

# 10. Noise

This can cause irreversible hearing damage and workplace accidents. Workers may be exposed not only to the noise that their work is making, but also to the background noise of other tasks on construction site. The main sources of noise include concrete breakers, explosives and engines. Where possible, the production of noise should be eliminated. This can be achieved by changing the construction or work method. Where elimination is not possible, noise should be controlled. Personal hearing protection should be used as a last degree of protection.

Employers must provide the employees with information, instruction and training and carry out health surveillance where necessary.  Employers are required to take specific action at certain action values. The action values relate to the levels of employees’ exposure to noise averaged over a working day or week (lower action value 80 dB, upper action value 85 dB); and the maximum noise (peak sound pressure) to which employees are exposed in a working day (lower action value 135 dB, upper action value 137 dB).

# 11. Vibration

Many jobs in construction involve the use of hand-held power tools. The vibration from such equipment can cause hand-arm vibration syndrome (HAVS). This condition affects the fingers, hands and arms and, in the long term, causes permanent damage. Eventually parts of the fingers go white and numb and there is a loss of touch. This condition is often called ‘vibration white finger’.

The whole-body vibration is caused by machines or vehicles that transmit vibration through the operator’s feet, buttocks and back into the body. Employers have to conduct a risk identification and assessment study at workplaces where vibration is a hazard. If the exposure is too high, employers should consider: alternative work procedures; more suitable work equipment; personal protective and supplementary equipment; maintenance programmes; workplace design; information on the proper use of vibrating machines; limiting the duration of exposure; and appropriate shift schedules with breaks.

Vibration damage to the fingers, hands or body is very much dose-related. The greater the exposure to vibration, the more likely there is to be damage. Other factors that can influence the degree of severity of hand-arm vibration syndrome include the grip, push and other forces used to guide and apply the vibrating equipment (the tighter the grip, the more vibration energy is transferred to the hands); the exposure pattern, length and frequency of work and subsequent rest periods; the hardness of the material being worked upon; and factors that can affect blood circulation, such as workplace temperature, smoking and individual susceptibility.

While all vibration can be damaging, it is perhaps logical to assume that the faster a machine vibrates, the more dangerous it is, whereas in fact it is lower-speed vibrations (those between about 2 and 64 vibrations per second) that cause the most damage. Some of the tools used in construction that are likely to result in high levels of exposure to vibration are: road and concrete breaking drills; concrete vibro thickeners and concrete pokers; plate vibrators; chisels (air or electric); compressor guns; pneumatic and percussive drills; angle grinders; sanders and other similar ‘rotary’ tools; abrasive wheels; cutting-off wheels and discs; power hammers and chisels.

Work equipment should be properly maintained in order to be balanced, without loose or worn out parts and dull blades/cutters, etc. It is important for workers to keep their hands warm to get a good flow of blood into the fingers by wearing gloves, having hot food or drinks, massaging the fingers; not smoking (as this can cause narrowing of the blood vessels).

# 12. Work-related stress

Pressure is often part of work and helps to keep people motivated. Excess, badly-managed exposure to pressure can lead to stress. Workers who experience stress, anxiety or depression are unlikely to perform effectively. This can be costly and – in safety-critical industries, such as construction – can have serious consequences. Work-related stress is not inevitable and can be tackled by sensible management.

The six major aspects of work that can be causes of stress across all jobs are: work demands, control over work, support from others, relationships at work, worker’s role and the effect of change.

The top five most stressful aspects of construction work are: having too much work to do in the time available; travelling or commuting; being responsible for the safety of others at work; working long hours; and having a dangerous job. Factors such as personal relationships, financial concerns, domestic issues and bereavement can also affect the ability to cope with pressure at work.

# References

- Health and Safety in Construction, third edition, 2006, ISBN 9780717661822;

- Avoidance of danger from overhead electric powerlines General Guidance Note GS6 (Third edition) HSE Books 1997, ISBN 0717613488;

- Control of Noise at Work Regulations 2005 SI 2005/1643 The Stationery Office 2005 ISBN 0110729846;

- Controlling noise at work. The Control of Noise at Work Regulations 2005. Guidance on Regulations L108 (Second edition) HSE Books 2005, ISBN 071766164 4;

- Electrical safety on construction sites HSG141 HSE Books 1995, ISBN 0717610004;

- European Agency for Safety and Health at Work, https://osha.europa.eu/en/sector/construction;

- Fire safety in construction work: Guidance for clients, designers and those managing and carrying out construction work involving significant fire risks HSG168 HSE Books 1997 ISBN 0717613321;

- Hand-arm vibration. The Control of Vibration at Work Regulations 2005. Guidance on Regulations L140 HSE Books 2005 ISBN 0717661253;

- Hand-arm vibration: Advice for employees Pocket card INDG296  (rev1) HSE Books 2005 ISBN 0717661180;

- Health and Safety Executive, http://www.hse.gov.uk/construction/index.htm;

- Manual handling. Manual Handling Operations Regulations 1992 (as amended). Guidance on Regulations L23 (Third edition) HSE Books 2004, ISBN 071762823X;

- Noise in construction: Further guidance on the Noise at Work Regulations 1989 Leaflet INDG127(rev) HSE Books 1994;

- Safe use of vehicles on construction sites: A guide for clients, designers, contractors, managers and workers involved with construction transport HSG144 HSE Books 1998 ISBN 071761610X;

- Safety and Health in Construction, International Labour Office, ISBN 92-2-107104-9,  Geneva. 1992