MODULE 5

HEALTH AND SAFETY SIGNS AND SIGNBOARDS

Project: 2013-1-BG1-LEO05-088722

Author:	CEEI-BURGOS

October, 2014

This project has been funded with support from the European Commission. This publication reflects the views only of the author, and the Commission cannot be held responsible for any use which may be made of the information contained therein.

TABLE OF CONTENTS

OVERVIEW	3

VISUAL TYPE SIGNS	4

SIGNBOARDS	4

Worksite enclosures	4

Safety colours according to hazard level	5

VISUAL SIGNS ARE CLASSIFIED AS	6

1.	Hazard signs	6

2.	Prohibition signs	8

3.	Mandatory signs	10

4.	Safety equipment signs	11

5.	Fire equipment signs	14

LIGHT SIGNALS	14

Characteristics and requirements of light signals	14

ACOUSTIC SIGNALS	15

Characteristics and requirements of acoustic signals	15

OLFACTORY SIGNALS	16

TACTILE SIGNAGE	16

CHOICE OF SIGNAGE TYPE	16

Risk of falls, collisions and blows	16

Hand signals	17

ANNEX 1	20

# OVERVIEW

Signage indicates the risks existing at a given location and time.

The minimum requirements for the provision of workplace safety and risk signage are established in Directive 92/58/EEC of the Council of 24 of June 1992. Each country was responsible for transposing said directive into their national laws.

In general, they are a set of stimuli affecting the performance of an individual. They show a situation that the worker may encounter during the task to be carried out, indicating how to respond to a certain risk. This does not eliminate the risk, meaning that other complementary techniques are required to mitigate the risk.

For signage to be effective, workers must receive adequate training in order to interpret it correctly.

This knowledge must be refreshed periodically through “refresher” courses. Signs can occur in many ways, and may be classified according to the senses:

# VISUAL TYPE SIGNS

Visually perceived signs that convey information through shapes and colours. Sight is the most-developed of our senses and therefore visual safety signs are the ones most used. We can distinguish:

# SIGNBOARDS

Panels with symbols giving us information about the risks and directions to be followed. To be effective, the information must be quick, and so the sign will be as simple and intuitive as possible. To achieve this pictograms are used, which tend to be as universal and easy to recognize as possible. This type of signage must be resistant to the passage of time.

The sign colour will immediately indicate what type of information we are receiving. The sign must be large enough to be seen at first glance, and comply with the provisions of Annex 3 of R.D.485/1997. Signs shall be placed in visible, well-lit places, at an appropriate height, within the field of view.

In order to avoid non-effectiveness of signage, signs must not be placed too close to each other.

Signs must be removed when the situation that justified them no longer applies.

## Worksite enclosures

Pursuant to Spanish Royal Decree 485/1997, construction sites must be completely fenced around the perimeter and duly indicated with signs posted at the main entrances showing the general risks of the site as well as the wording "no entry - authorised persons only" to prevent access by non-authorised persons.

This situation must continue as long as the situation that motivated the signposting continues and it must be adapted to the environment and the different phases of the work.

Example of signboard:

### Safety colours according to hazard level

| COLOUR | MEANING | INDICATIONS |
| --- | --- | --- |
| RED | Prohibition sign | Dangerous behaviour |
|  | Danger Alarm | Stop |
|  | Fire equipment and materials | Identification and location of fire |
| YELLOW | Hazard | Caution |
| BLUE | Mandatory sign | Mandatory protection equipment |
| GREEN | Emergency sign | Doors, exits, rescue |
|  | Safe conditions | Back to normality |

Contrast colours: To be as clearly visible as possible and attract the necessary attention, so they are given the importance required, safety pictograms are designed using a series of colours and certain contrasts.

| COLOUR | CONTRAST COLOUR |
| --- | --- |
| RED | WHITE |
| YELLOW | BLACK |
| BLUE | WHITE |
| GREEN | WHITE |

# VISUAL SIGNS ARE CLASSIFIED AS

- Hazard signs

Triangular shape. Black pictogram on yellow background, black borders

DANGER

FORK LIFTS

Hazard signs: They must be triangular.  The pictogram will be black on a yellow background (yellow must cover at least 50% of the sign surface), black borders.

As an exception, the background of the sign for “'harmful or irritant substances” will be orange, instead of yellow, to avoid confusion with other similar signs used for road traffic regulation.

| COLOUR |  |  | SAFETY SIGN | MEANING |
| --- | --- | --- | --- | --- |
| SYMBOL | SAFETY | CONTRAST |  |  |
| Black | Yellow | Black |  | Flammable risk |
| Black | Yellow | Black |  | Risk of explosion |

Hazard signs not included in the R.D. and commonly used on site.

## Prohibition signs

They will be round. Black pictogram on white background, red borders and band.

DO NOT     ENTER

Prohibition signs: They will be round. The pictogram must be black with a white background, borders and band (45º diagonally downwards from left to right). Red must cover at least 35% of the sign surface.

| COLOUR |  |  | SAFETY SIGN | MEANING |
| --- | --- | --- | --- | --- |
| SYMBOL | SAFETY | CONTRAST |  |  |
| Black | Red | White |  | No smoking |
| Black | Red | White |  | No access for pedestrians |

No smoking                              No naked lights                    No access for pedestrians             Do not extinguish with water

Strictly no admittance                           Not drinking water                          No fork lifts                                     Do not touch

Prohibition signs not included in the R.D. and frequently used on site.

## Mandatory signs

They must be round. White pictogram on blue background.

HEAD PROTECTION MUST BE

WORN

Mandatory signs: They must be round. The pictogram must be white on blue background with white contrast.

| COLOUR |  |  | SAFETY SIGN | MEANING |
| --- | --- | --- | --- | --- |
| SYMBOL | SAFETY | CONTRAST |  |  |
| White | Blue | White |  | Head protection must be worn |
| White | Blue | White |  | Foot protection must be worn |

Mandatory signs not included in the R.D. and frequently used on site.

## Safety equipment signs

They must be rectangular or square. The pictogram must be white on a green background (green must cover at least 50% of the sign surface).

Safety or emergency signs must be rectangular or square. White pictogram on green background.

FIRST AID

This First Aid sign is prohibited in some countries because of religious symbols and is replaced by a Red Crescent or Red Diamond

The red diamond sign is used for first aid purposes in countries such as:

- Arab countries (Arabian Peninsula)

- Arab countries outside Arabia

- Muslim non-Arab countries

- Countries with a significant Muslim minority

ARAB COUNTRIES (ARABIAN PENINSULA)

ARAB COUNTRIES OUTSIDE ARABIA

MUSLIM NON-ARAB COUNTRIES

COUNTRIES WITH A SIGNIFICANT MUSLIM MINORITY

| COLOUR |  |  | SAFETY SIGN | MEANING |
| --- | --- | --- | --- | --- |
| SYMBOL | SAFETY | CONTRAST |  |  |
| White | Green | White |  | Escape route |
| White | Green | White |  | Stretcher |

Direction of escape route

First aid              Stretcher        Emergency       Eye wash station

shower

Emergency telephone

Direction /  Escape route

These panels may be luminescent when located in places where the sign is difficult to see.

## Fire equipment signs

They must be rectangular or square. The pictograms will be white on a red background (Red must cover at least 50% of the sign surface).

Direction to be followed in case of emergency

Fire hose reel                                   Fire ladder                              Fire extinguisher                  Fire telephone

These panels may be luminescent or photo luminescent when located in places where the sign is difficult to see.

# LIGHT SIGNALS

There are certain operations in which information is displayed by a light signal, for example, moving machinery may have a flashing, rotating, amber light.

## Characteristics and requirements of light signals

The light must cause an appropriate luminous contrast with its surroundings, in accordance with the terms of use laid down. It must be clearly visible without dazzling others.

The luminous surface transmitting the signal must be a uniform colour or have a pictogram on a certain background.

If a device can give off a continuous and flashing signal, the flashing signal will be used to indicate, with respect to the continuous signal, a higher degree of danger or greater urgency of the action required.

Two luminous signals cannot be used at the same time in order to avoid confusion, nor shall a luminous signal near another one give off only a slightly different luminous signal.

When a flashing luminous signal is used, the duration and frequency of the flashes must allow the correct identification of the message, avoiding it being perceived as continuous or confused with other light signals.

Light signal devices for use in the event of serious danger shall subject to special maintenance or be fitted with an auxiliary lamp.

# ACOUSTIC SIGNALS

Acoustic signals are caught by the ear. They are emitted by appropriate devices, without the intervention of the human voice. They may be encoded type sounds. This type of signage is infrequent and requires special training for the worker.

## Characteristics and requirements of acoustic signals

The acoustic signal must have a noise level higher than the level of ambient noise so that it is clearly audible, without being too annoying. An acoustic signal should not be used when the ambient noise is too intense.

The tone of the acoustic signal, or in the case of intermittent signals, the duration, interval and grouping of the signals must allow their correct identification and clear distinction with regard to other acoustic signals or ambient noise. Two acoustic signals should not be used simultaneously.

If a device can emit acoustic signals with a variable or intermittent tone or intensity, or with a continuous tone or intensity, the former shall be used to indicate, as opposed to the latter, a higher danger level or a more urgent need of action required.

The sound of an evacuation signal shall be continuous.

# OLFACTORY SIGNALS

They use smells that convey some kind of information. They imply the need for adequate training of the worker. The main setback is that an olfactory signal may be disguised by the environment. Gaseous fuels for domestic use are odorized to identify the presence of leaks.

# TACTILE SIGNAGE

These signs are perceived by touch, our hands being the most sensitive to these signs. This type of signage is not very frequent (for example, making the controls of a machine rougher makes them more difficult to use).

# CHOICE OF SIGNAGE TYPE

The choice of the type of sign, as well as the number and location of the signs or signage devices to be used in each case will be done so the signage is as efficient as possible, taking into account:

• The characteristics of the signage.

• The risks, items or circumstances that are to be indicated.

• The size of the area to be covered.

• The number of workers affected.

## Risk of falls, collisions and blows

For signage of uneven levels, obstacles or other elements that pose a risk of fall, collision or blow, either the corresponding panel described above or a safety colour may be chosen, with equal efficiency, or both may be used complementarily.

The delimitation of those work premise areas to which the worker has access, where risks of people falling, falling objects, collisions or blows exist, shall be done using a safety colour.

The colour-coded signage referred to in the two previous paragraphs shall consist of alternating yellow and black stripes. The strips must have an approximate angle of 45º and similar dimensions to the model below.

## Hand signals

A hand signal must be precise, simple, easy to make and understand, and clearly distinguishable from any other hand signal.

The use of both arms at the same time shall be one in a symmetrical manner and for a single hand signal.

The hand signals used, with regard to the characteristics listed above, may vary or be more detailed, provided that their meaning and understanding are at least equivalent.

The signaller must be able to visually follow the manoeuvres without running any danger.

The signaller must be exclusively in charge of directing operations and the safety of workers in the vicinity.

The operator shall suspend the manoeuvre being performed to request new instructions when the orders received cannot be carried out with the necessary safety guarantees.

The signaller must be easily recognizable by the operator.

The signaller shall wear one or more appropriate identification elements such as a jacket, sleeves, armband, or helmet and, when necessary, auxiliary devices. The identification elements mentioned must be brightly coloured, and, if possible, the same for all the elements, and used exclusively by the signaller.

|  | - General gestures |  |
| --- | --- | --- |
| Meaning | Description | Illustration |
| Beginning: Warning Take command | Both arms extended horizontally with palms forward. |  |
| Stop: Interruption End of the movement | Right arm extended upwards, palm forward. |  |
| End of operations | Both hands together at the height of the chest |  |

|  |  | - Vertical movements |  |  |
| --- | --- | --- | --- | --- |
| Meaning |  | Description | Illustration |  |
| Hoist |  | Right arm extended upwards, palm forward moving slowly in a circle. |  |  |
| Lower |  | Right arm extended down, palm to the inside moving slowly in a circle. |  |  |
| Vertical distance |  | Hands indicate the distance. |  |  |
|  | - Horizontal movements |  |  |  |
| Meaning | Description |  |  | Illustration |
| Forward | Both arms bent, palms toward the inside with the forearms moving slowly towards the body. |  |  |  |
| Back | Both arms bent, palms toward the outside with the forearms moving slowly away from the body. |  |  |  |
| To the right: With respect to the person responsible for the signals. | Right arm extended more or less horizontally, right palm down making small, slow movements indicating the direction. |  |  |  |
| To the left: With respect to the person responsible for the signals. | Left arm extended more or less horizontally, left palm down making small, slow movements indicating the direction. |  |  |  |
| Horizontal distance | Hands indicate the distance. |  |  |  |

|  | - Danger |  |
| --- | --- | --- |
| Meaning | Description | Illustration |
| Danger: Stop or emergency stop | Both arms extended upwards, palms forward. |  |
| Quick | The coded gestures referring to the movements are done quickly |  |
| Slowly | The coded gestures referring to the movements are done very slowly. |  |

# ANNEX 1

Meaning of symbols

Regulation EC 1272/2008 on the classification, labelling and packaging of substances and mixtures introduces changes to the pictograms on labels used for chemical products. The new hazard pictograms are diamond-shaped. They feature a symbol in black on a white background with a red frame that is thick enough to be visible. One of the following warning words, which refer to the severity of the hazard, appears next to the hazard pictogram:

- DANGER, for the most hazardous categories.

- WARNING, for the less hazardous categories

The new way to label pesticides has been in for since December 2010, yet the transition period shall remain effective until the year 2015 during which time the old and new classification and labelling systems may both be used. This means phytosanitary users may find products on the market with the new pictograms or the old ones. The equivalencies are shown in the table on the page.

Explosive

Solid, liquid, doughy or gelatinous substances and preparations which, even in the absence of oxygen in the air, may produce an exothermic reaction with the quick formation of gases which, under certain test conditions, may detonate, deflagrate quickly or explode under the effect of heat, in the event of partial confinement.

Combustive agent (O)

Substances and preparations which, when in contact with other substances, especially with inflammable substances, produce a strong exothermic reaction.

Inflammable products (F):

Liquid substances and preparations with a low flash point.

Easily inflammable products:

Substances and preparations:

- That can heat up and become inflamed in the  air room temperature without any added energy, or

- Solids that can easily become inflamed after brief contact with a source of inflammation and continue burning or smouldering once said source is removed, or

- Liquids with a very low flash point, or

- Which when in contact with water or moist air, release extremely inflammable gases in hazardous quantities.

Extremely inflammable products:

Liquid substances and preparations with an extremely low flash point and a low boiling point, and gas substances and preparations which, at normal temperature and pressure, are inflammable when in contact with the air.

Corrosive products (C):

Substances and preparations which, when in contact with live tissues, can exercise destructive action on them.

Irritants (Xi):

Non corrosive substances and preparations which, when in brief, prolonged or repeated contact with the skin or mucous membranes, may cause an inflammatory reaction.

Harmful products (Xn):

Substances and preparations which, due to inhalation, ingestion or skin penetration, may cause acute or chronic effects and even death.

Toxic products (T):

Substances and preparations which, due to inhalation, ingestion or skin penetration in small quantities, may cause acute or chronic effects and even death.

Very toxic products:

Substances and preparations which, due to inhalation, ingestion or skin penetration in very small quantities, may cause acute or chronic effects and even death.

Sensitizing products:

Substances and preparations which, due to inhalation or skin penetration, may cause a hypersensitivity reaction in such manner that later exposure to the same substance or preparation gives rise to characteristic negative effects.

Carcinogenic products:

Substances and preparations which, due to inhalation, ingestion or skin penetration, may cause cancer or increase the frequency thereof.

Mutagenic products:

Substances and preparations which, due to inhalation, ingestion or skin penetration, may cause hereditary genetic alterations or increase the frequency thereof.

Hazardous for the environment (N):

Substances and preparations that present or may present an immediate or future hazard to one or more components of the environment.

| Pictograms as per regulation before EC 1272/2008 | Pictograms as per Regulation EC 1272/2008 |
| --- | --- |