MODULE 6

SIGNALS AND FIRST AID

Project: 2013-1-BG1-LEO05-088722

Author:	BEST Institut für berufsbezogene Weiterbildung und Personaltraining GmbH

Austria

October, 2014

This project has been funded with support from the European Commission. This publication reflects the views only of the author, and the Commission cannot be held responsible for any use which may be made of the information contained therein.

TABLE OF CONTENTS

OVERVIEW	3

FIRST AID	4

HEALTH AND SAFETY SIGNS AND SIGNALS	7

Procedures to provide first aid	8

Indicative first aid actions	11

ANNEX I: Dangerous substances in a construction site	15

# OVERVIEW

This module provides an introduction to first aid, including definitions and specific provisions. In addition, an analysis of concrete information related to the provision of first aid in the construction site is provided, as well as the basic principles of first aid per category of accident and/ or injury.

All first aid provisions are indispensable to be in place and respected in all construction sites. The responsibility for first aid regulations and personnel lies belongs to the employer of the personnel who works on site, who has the obligation of making sure that first aid materials/ equipment exist in all the places of work and to the extent that they cover sufficiently the whole construction site. Therefore, depending on the size of the site, but also on the specific hazards involved with the construction at question, the first aid procedures and material may vary. In addition, it is the obligation of the employer to make sure the appropriate number of properly trained personnel for the provision of first aid is present, during working hours.

The information provided below does not substitute any form of training which is crucial for the provision of first aid in construction sites. However, it aims to inform the target groups of the eGuide project, i.e. VET students in construction and related sectors, VET trainers and tutors in construction and related industries and SME managers and employees in construction companies about the minimum standards necessary for first aid in any construction site.

This module starts with the definition of first aid, along with the related general requirements for every construction site. The first aid room and kit requirements are also presented, along with the most common types of hazards that are likely to occur at a construction site. Basic principles per injury or accident of first aid provision are also included. Finally, the Annexes relate to the most common dangerous substances on site and the module Glossary.

A set of questions for self-assessment are available, as well as a case study, which relates to validating the comprehension of the first aid requirements and principles at a construction site.

# FIRST AID

First aid is the immediate and basic care given to an injured or sick person before a doctor, other health professional or emergency services take over their treatment. 
When providing first aid assistance, the objectives are to:

- Preserve life;

- Prevent the injury or illness from becoming worse;

- Promote recovery.

First aid requirements at work fall into three categories:

- Suitably stocked first aid kits and facilities

- Where needed, an appropriate number of suitably trained first aiders

- Information for employees about first aid arrangements.

Health and safety Times-The Construction Workers Heath Trust

Employers are obliged to have available first aid equipment, facilities, trained personnel and first aid procedures on display in all workplaces, especially at construction sites.

As far as the equipment is concerned, a first aid station should be established, accessible at all times. The employer should make sure that:

- The station runs under the supervision and responsibility of the person who is assigned with the provision of first aid services. He/ she should make sure that the station is cleaned regularly and carefully and that stock is replenished.

- The station is always available during working hours, as well as overtime.

- The station is a room dedicated for this purpose alone.

- It is located on the ground floor, as close as possible to the entrance and bears the appropriate signs so that emergency services will identify it at once.

- It is being ventilated and properly illuminated.

- It is large enough to fit, at least, a bed and people working around it, as well as a chair.

- The entrance of the station is wide enough to fit a stretcher, a wheelchair or similar.

- It contains all the necessary first aid facilities and equipment, has waterproof floor covering and is well maintained. The surfaces can be easily cleaned and provisions are made for disposing garbage, laundering sheets, blankets, towels, etc.

- The station has provisions for cold and hot running water.

- There are enough cupboards for storing first aid supplies.

When there are vehicles in the workplace, each one should be equipped with a first aid kit. Similarly, a first aid kit is obligatory when workers are operating heavy construction and maintenance equipment and the first aid station is not close. The kit should be inspected on a quarterly basis and the contents replaced at a regular level. The following factors should be considered in relation to the provision of appropriate first aid kits:

Accidental Health and Safety-Industry Health Kits

Contents should be appropriate for the workplace hazards identified and the types of illness/ injury that is likely to happen.

- Number of kits varies depending on the size of the workplace, the use of vehicles, the structure of the workplace, etc.

- First aid kits should be located close to areas where there is a likely risk of injury/illness occurring. They should be clearly visible and easily accessible. First aid kits must be provided for persons working in remote areas or in vehicles where access to accident and emergency services may be limited.

- The first aid kit should be clearly identifiable, with a white cross on a green background prominently displayed on the outside.

- A list of contents should be provided with the kit. Names and locations of trained first aiders should also be displayed, along with relevant telephone numbers, including emergency lines.

Industrial Kits and Cabinets-Medique Products

The first aid kit should be managed by a trained first aider. This person should be responsible for assessing kit requirements, checking and replenishing contents, and ensuring kits are available and not locked whenever employees are at work.

Below is a list with the recommended contents of a first aid kit at a workplace:

- Individually wrapped moist wipes or saline solution

- 20 individually wrapped sterile adhesive dressings (assorted sizes), appropriate to the type of work

- Two sterile eye pads

- Two individually wrapped triangular bandages (sterile)

- Clasps or safety pins to tie bandages

- Two stretch bandages

- Six medium sized, individually wrapped wound dressings

- Two large sterile individually wrapped unmedicated wound dressings

- Two pairs of disposable gloves

- One resuscitation mask

- Scissors

- Adhesive strips or band aids for minor wound dressing

- Non-allergic adhesive tape

- Disposable aprons

- Forceps or tweezers to remove foreign bodies

- Plastic bags for waste disposal

- Hand sanitiser

In case that first aid is required, it is the obligation of the employer to make sure that this is done by trained and knowledgeable workers, who have undergone the appropriate type of training. This means that:

- They hold a current first aid certificate, issued after the successful completion of a first aid course (certified by a public authority or accredited organisation) or

- They hold a current occupational first aid certificate, issued successful completion of a relevant course (certified by a public authority or an accredited organisation) or

- Be a registered nurse or

- A medical practitioner.

How to handle construction site safely-GroundReport.com

It should be noted that the skills and competencies of trained personnel who is in the position to provide first aid services should maintained at a level appropriate for the workplace where they are employed. For example, first aiders in foundries must be particularly skilled in treating burns, whereas first aiders in the construction industry must be skilled in monitoring symptoms of employees that may be injured due to falls or laceration. Subsequently, additional training might be considered as necessary when there’s a change of workplace with different hazards and potential injuries and/ or illnesses.

The most common hazards at a construction site are summarised below (the table does not include all hazards, but It provides examples of typical problems created by workplace hazards, and some of the resultant injuries and illnesses).

| Hazard | Typical Problems | Typical injury/ illness requiring first aid |
| --- | --- | --- |
| Manual handling | Overexertion/Repetitive movement | Sprains, strains, fractures |
| Falls | Falls from heights, slips and trips on uneven surfaces | Fractures, bruises, cuts, dislocations, concussion |
| Electricity | Contact with electrical current | Shock, burns, loss of consciousness, cardiac arrest |
| Plant | Being hit by projectiles, striking, objects, being caught in machinery, overturning vehicles | Cuts, bruises, dislocations, fractures, amputation, eye damage |
| Hazardous substances | Exposure to chemicals, e.g. solvents, acids, hydrocarbons | Dizziness, vomiting, dermatitis, respiratory problems, burns to skin or eyes |
| Temperature, UV radiation | Effects of heat or cold from weather or work environment | Sunburn, frostbite, heat stress, heat stroke, hypothermia |
| Biological | Allergens, needle stick, exposure to infectious agents | Severe allergic reaction, injuries, skin rash, infection |
| Occupational violence | Intimidation, conflict, physical assault | Nausea, shock, collapse, physical injuries |

More information on hazardous substances can be found at “Managing and Controlling Hazards in Construction Activities” of the Rapid Results College website (http://www.rrc.co.uk/nlpdfs/Summer08/Articles/NCC1u08.1.3.pdf).

# HEALTH AND SAFETY SIGNS AND SIGNALS

First aid signs used in Europe are

In Turkey, until recently, different signs were available. However, the obligatory health and safety signs and signals are no longer different from the European standards, since the new the 2012 OHS Law enactment. The regulation for H&S Signs and Signals has been renewed and enacted on 11 September 2013 according to the Article 30 (Regulations related to occupational health and safety) of the OHS Law No.6331 and parallel to the European Parliament and Council Directive numbered 92/58/EEC and dated on 24.06.1992.

As soon as the general use and maintenance of signs is no different than the EU standards, it is presumed that the provision of all details in this report is not required. All the use of health and safety signs and signboards such as; prohibition, warning, mandatory and emergency escape signs brought to the same standards used in Europe except the main first-aid and related signs. While the European standards suggest a plus sign for-first aid, the new regulation in Turkey do not impose to use this sign because there is a general recognition of a crescent sign for first-aid purpose. The other related signs for stretcher, safety shower and eyewash is also being used without the plus sign as shown in the below table.

| TURKISH FIRST-AID SIGNS |  | EUROPEAN FIRST-AID SIGNS |  |
| --- | --- | --- | --- |
| First-aid | Stretcher | First-aid | Stretcher |
| Safety Shower | Eyewash | Safety Shower | Eyewash |
| Emergency telephone for first-aid or escape |  | Emergency telephone for first-aid or escape |  |

## Procedures to provide first aid

As already presented, first aid is the provision of immediate basic medical assistance to a sick or injured person, until professional medical treatment can be provided. First aid includes a series of simple and (in most cases) life-saving procedures which should be provided as soon as the injury happens of the illness persists and worsens. These procedures can be execute by a suitably trained individual, who bears officially this responsibility, with minimal equipment that should be available in the workplace and especially in construction sites.

The most important principle about first aid is that it should be provided immediately by the person who is responsible to provide it. Any action taken needs to be careful and targeted to save or relieve the person who is sick or injured. Therefore, it is mostly important that the first aid carer remains calm at all times.

The first priority is to make sure that there is no imminent danger for the carer and the bystanders. A risk assessment is therefore very important to ascertain that no other people will get injured or affected. For example, any risk of explosion, fire, contamination, fall, injuries by blunt objects, electrocution, etc. should be decreased to a minimum. It is often mostly helpful to evacuate the surrounding area, both for avoiding more injuries or contamination, as well as for creating the conditions for the carer to remain calm and concentrated to provide first aid assistance.

Before providing any type of assistance, first aiders should wash their hands with soap or use an alcohol-based hand sanitiser. They should also wear protective gloves and any other protective equipment deemed necessary, depending on the emergency they are called to address. Such equipment includes eye protection, a mask and protective clothing. Additional procedures which relate to the safety of other workers include:

- Hygiene practices

- Handling and disposal of sharp objects, as well as material used for the provision of first aid

- Handling and disposal of waste

- Cleaning and sanitisation of surfaces and reusable equipment

- Actions to be taken if there is contact with blood or body substances, blunt objects, contaminated objects, etc.

In addition to the above, the first aider or a person who is present at the site should immediately call emergency (in Europe: 112), particularly in case of heat stroke, heart attack, fractures with open wounds etc.

Clear details on what happened should be provided, such as where the incident happened, how many people were involved, which are the main casualties, which are the main injuries, etc.

Common procedures to know before emergency wards arrive are the "ABC"s of first aid to help in critical life-saving situation and must be rendered before treatment of less serious injuries.

Airway, Breathing, Circulation Disability, Exposure (ABCDE) is an approach for initial assessment and treatment, applicable in all clinical emergencies. It is widely accepted by experts in emergency medicine and it has been evidenced to improve outcomes by helping health care professionals focusing on the most life-threatening clinical problems. The aims of the ABCDE approach are to:

- Provide life-saving treatment

- Break down complex clinical situations into more manageable parts

- Serve as an assessment and treatment algorithm

- Establish common situational awareness among all treatment providers

- Buy time to establish a final diagnosis and treatment.

A

In principle, life threatening airway problems are assessed and treated. If the patient (injured or sick person) responds in a normal voice, then the airway is clear. Signs of partially obstructed airway include changed voice, heavy and/ or noisy breathing. Also, a reduced level of consciousness is a common cause of airway obstruction, partial or complete.

Untreated airway obstruction can rapidly lead to cardiac arrest. A head-tilt and chin-lift manoeuver to open the airway is in most cases an effective method to open the airway. In cases of obstruction (such as blood or vomit), it is advices to give five back blows alternating with five abdominal thrusts until obstruction is cleared to conscious patients. In the case of unconscious patient, then professional help is required for cardiopulmonary resuscitation.

B

Secondly, respiratory rate should be determined. The first aider should inspect movements of the thoracic wall for symmetry and use of auxiliary respiratory muscles. Cyanosis, distended neck veins and lateralisation of the trachea can be identified by a trained firs aider. If breathing is insufficient, assisted ventilation must be provided, by rescue breaths.

C

Circulation sufficiency can be identified through colour changes, sweating and decreased level of consciousness. If a stethoscope is available, heart auscultation should be performed.

D

When it comes to disability the first aider should assess the level of consciousness, using AVPU method, according to which the patient is graded as alert (A), voice responsive (V), pain responsive (P) or unresponsive (U). Limb movements should be inspected to evaluate potential signs of lateralisation. The best immediate treatment for patients with a primary cerebral condition is stabilisation of the airway, breathing and circulation. Pupillary light reflexes should be evaluated and blood glucose measured. A decreased level of consciousness due to low blood glucose can be quickly corrected with oral or infused glucose.

E

Finally, the first aider should look for any clues to explain the condition of the patient. In particular, they should observe for any signs of trauma, bleeding, skin reactions (rashes), needle marks, etc. Clothing should be removed to allow for a thorough physical examination. Also, the body temperature should be checked, in the absence of a thermometer, through feeling the skin.

## Indicative first aid actions

In the sections that follow, four typical cases are presented on first aid provision until professional medical help arrives:

First aid until arrival of emergency service

In case of heat stroke/ heat exhaustion, the person should be moved out of the sun and into a shady or air-conditioned space. Then emergency should be called. The person should receive assistance to cool, such as being covered with damp sheets or ice packs to the armpits, neck, and groin or by cool water spraying, direct air with a fan or any object which could generate cool air (e.g. newspaper, magazine, etc.). Finally, cool water should be given or another non-alcoholic beverage, without caffeine.

If the person vomits, they should be turned on their side, to avoid air obstruction and make sure that the airway remains open.

First aid in case of Cuts or Lacerations

In cases of cuts or lacerations, the first aid provider should locate the injury and make sure that it is a minor injury. In this case, they should attempt to stop the bleeding by applying direct pressure on the area. Then the area needs to be cleaned with warm water and gentle soap.

An antibiotic ointment to reduce the chance of infection should be applied, along with a sterile, non-adhesive bandage or clean cloth, or adhesive strip for small injuries. In the event that the patient is allergic to the antibiotic provided, the application of the ointment should be stopped.

If the cut is bleeding severely, or blood is spurting out or the bleeding cannot be stopped after 10 minutes of firm and steady pressure then usually stitches need to be applied. Such a cut should be repaired within 6 hours from the moment that the injury happened. In these cases, emergency assistance is obligatory.

First aid in case of burns

The principle action to be taken is to immediately stop the source of burning, by putting off the fire or reposition the person to stop their contact with the flame (hot liquid, stream or other material). The person should be assisted to stop, drop and roll to put out the flames that might be burning their body.

Any smouldering material, hot or burned clothing should be removed. In the event that clothing sticks to the skin, then the first aider should cut or tear the clothes around these parts.

If the person has suffered first degree burns, which affect the top layer of the skin, the first aider should cool the burn, by holding the burned skin under cool running water or immerse the body part which has suffered the burn in cool water, until the pain subsides. If no running water is not available, compresses should be used. The burn should be protected with a sterile, non-adhesive bandage or clean cloth, without applying butter or ointments, since they may cause infection. A pain killer should also be provided, when necessary, but always bearing in mind potential allergies or any medication issues.

If the person has signs of infection, such as increased pain, redness and swelling, fever or oozing, then the patient needs to seek professional medical care.

For second degree burns, which affect the top two layers of the skin, the first aider should cool the burn by immersing the burned body part in cool water 10 or 15 minutes, or use compresses if running water is not available. No ice should be applied, as it may cause further damage by lowering the body temperature too much. Also, blisters should not be broken and no ointment or butter should be applied, as they may cause infection.

The burn should be covered loosely with a sterile, non-adhesive bandage and secured in place with gauze or tape. If the patient does not suffer from another injury (head, neck, leg), they should receive assistance to lay down, their feet should be elevated, as well as their burn area above heart level. For more comfort, a blanket should be used to cover the person. Medical care should be sought, so that the burn severity may be assessed and the appropriate medication can be administered.

When it comes to third degree burns, an ambulance should be immediately called. Until arrival, the burn area should be covered loosely with sterile, non-adhesive bandage or, for large areas, a sheet or other material that that will not leave lint in wound.

If the burn is on the hands and/ or legs, fingers and toes should be separated with sterile dressings. To prevent shock the person should again receive assistance to lay flat, as described above, unless they also suffer another injury.

If the person has an airway burn, they should be helped to lay down, but without a pillow, since the airway might close. If the person has suffered a face burn, they should get help stay up. Pulse and breathing should be constantly monitored, until the ambulance arrives.

Fracture Treatment

Fractures are broken or cracked bone(s), which are due to pressure applied to the bone and may or may not result in displacement of bone fragments. There are five types of fractures:

- Closed fractures: no skin break

- Open fractures: skin break causing an open wound

- Complicated fractures, which include damage to adjacent organs

- Stress fracture: hairline crack due to repeated stress

- Greenstick fracture to children’s flexible bones.

The most common symptoms of fractures are severe pain, difficulty in movement, swelling and/ or bruising and/ or bleeding, deformity or abnormal twist and tenderness when applying pressure.

If a bone has broken through the skin, the first aid provider shouldn’t try to push it back into place. Instead, they should call emergency and until it arrives, the area should be covered with a clean bandage or cloth and the bleeding should be attempted to be controlled, by applying steady, direct pressure for 15 minutes and elevate the wound. If blood soaks through, another cloth over the first one should be applied and immediate medical attention should be sought. At all times the patient should be reassured and comforted. The broken wound should be immobilised, while the first aider searches whether there are other injuries as well. The area should not be massaged and no attempt should be made to straighten the broken bone. Likewise, the patient should not be moved without having support at the broken bone.

A commonly used method is the RICE therapy:

Rest ankle by having the person stay off of it. Use crutches if necessary.

Ice area: use an ice pack or wrap the ice in a clean cloth. Do not place ice directly against the skin.

Compress by wrapping ankle lightly (not tightly) with an "ace" bandage or elastic ankle brace. Do not try to align the bones.

Elevate ankle above heart level.

First aid training are offered by the red cross e.g., who also offers online details: http://www.redcrossfirstaidtraining.co.uk/

In Austria there is also a self-assessment for generic first aid available: https://kurse.erstehilfe.at/

Hazard assessment is equally important as any step in the first aid provision process. In particular, one should consider:

- How often are workers exposed to any of the hazards identified?

- For how long are workers exposed to the hazards?

- What is the expected severity of the injury or illness that might be the result of the hazard?

- Which are the preventive measures that can be applied to minimise either the hazard or the exposure of the workers to it.

Another important element to consider related to first aid in the workplace is record keeping. Given that personal health information is confidential, the person assigned to provide first aid services should consider developing a record keeping and reporting system, for incidents and accidents that have occurred in the workplace. The information which should (at least) be recorded is:

- date, name, time;

- description of symptoms;

- the treatment provided;

- any referral arrangements (e.g. ambulance, hospital, medical service).

To record and have the possibility to access the information aforementioned, special forms should be developed and located at the first aid stations. If a person is transferred to a hospital or medical service, a copy of the first aid record should accompany them. Employees should be given a copy of their first aid record, and must have access to it on request.

# ANNEX I: Dangerous substances in a construction site

On the 22nd of March 2012 the European Commission issued an indicative list of regulated dangerous substances possibly associated with construction products (CPD = DS 041/051 rev.12). With this document the Commission and its Expert Group on Dangerous Substances in the field of Construction Products (EGDS) provide guidance in the construction sector concerning substances and parameters, which should be taken into consideration when assessing the availability of test methods and the need for developing harmonised test methods. This updated document substitutes the previous publicly available version of the indicative list (DS 041/051 rev.9) of 29th May 2009 (CEN/TC 351 N 0230rev). The complete list is available on the website of DG Enterprise and Industry and can be accessed through this link: http://ec.europa.eu/enterprise/sectors/construction/cp-ds/index_en.htm.

The substances used and/ or produced at a construction site can come in different forms, such as:

- Chemicals;

- Fumes;

- Dusts;

- Vapours;

- Mists;

- Nanotechnology;

- Gases;

- Asphyxiating gases;

Asbestos Disposal Oxford and Abingdon-Amity Group

Biological Elements

When there is a harmful substance at the workplace falling under any of the categories mentioned above, the workers might by exposed by:

- Breathing in gases, fumes, dust or mist: Once breathed in, some substances can attack the nose, throat or lungs while others get into the body through the lungs and harm other parts of the body, e.g. the liver;

- Skin contact: Some substances damage skin, while others pass through it and damage other parts of the body. Skin gets contaminated:

- by direct contact with the substance, eg if you touch it or dip your hands in it;

- by splashing;

- by substances landing on the skin, eg airborne dust;

- by contact with contaminated surfaces – this includes contact with contamination inside protective gloves;

- Swallowing: People transfer chemicals from their hands to their mouths by eating, smoking etc. without washing first;

- Contact with the eyes: Some vapours, gases and dusts are irritating to eyes. Caustic fluid splashes can damage eyesight permanently;

- Skin puncture: Risks from skin puncture such as butchery or needle stick injuries are rare, but can involve infections or very harmful substances, e.g. drugs.

Health and Construction-Recognition, Evaluation, Control

The symbols for hazardous substances are the ones below:

The table below displays a categorisation of dangerous agents. A material is considered as dangerous if it complies with at least one of the characteristics mentioned below

http://www.webmd.com/first-aid/thermal-heat-or-fire-burns-treatment

http://www.webmd.com/first-aid/thermal-heat-or-fire-burns-treatment

http://www.webmd.com/first-aid/thermal-heat-or-fire-burns-treatment

First Aid in the Workplace, NSW Health and Safety Guide, 2001

http://en.wikipedia.org/wiki/First_aid

En.wikipedia.org

International Journal of General Medicine, 2012, 5: 117-121

Working with substances hazardous to health, Health Safety Executive,

Brochure of Occupational Health and Safety, Federal Ministry of Labour, Social Affairs and Consumer Protection